InterruptSharingwithKITL:

- Making use of registry editor tool to ensure that our devices (MCS9865, MCS9901) com port doesn’t share the KITL irq.

- If they share the irq with KITL then we can’t have those com ports working.

- Make sure we have different irq than KITL(Realtek card)

Using the registry editor for WinCE5.0:

1) Tools->Remote registry Editor ->Ok

2) Default Device->HKEY_LOCAL_MACHINE->Drivers->BuiltIn->PCI->Instance

3) Now check the KITL irq by selecting KITL.

4) Note the Irq.

Example: KITL     has Irq=9

5) Check all pci driver instances of our device (MCS9865/MCS9901):

Example:

MCS9901Ser1    has Irq=10

MCS9901Ser2    has Irq=11

MCS9902ISA1   has Irq=10

As the irq of all pci driver instances are different than that of KITL, all ports of our device works fine.

Using the registry editor for WinCE6.0:

- Target->Remote Tools->Registry Editor

Note: Repeat the steps, step: 2 to step: 5 from above topic i.e. Using registry editor

For WinCE5.0

Note:

The KITL irq is not sharable, if Realtek card is used for KITL see the link for

Reference below:

http://www.tech-archive.net/Archive/WindowsCE/microsoft.public.windowsce.platbuilder/2004-03/0755.html

I have tested MCS9865 and MCS9901 devices at different slots of my PC and included the results at:

1) I have checked the MCS9865 in all three slots here are my observation:

|  | Slot Number 1 | IRQ | Remarks |
| --- | --- | --- | --- |
| Devices | KITL(Realtek) | 11 |  |
|  | Ser1 | 10 |  |
|  | Ser2 | 9 |  |
|  | SerIsa | 11 | SerialIsa ports doesn’t work, fail at setting baud rate |

|  | Slot Number 2 | IRQ | Remarks |
| --- | --- | --- | --- |
| Devices | KITL(Realtek) | 11 | Here both the Isa and the standard com ports work fine |
|  | Ser1 | 9 |  |
|  | Ser2 | 10 |  |
|  | SerIsa | 9 |  |

|  | Slot Number 3 | IRQ | Remarks |
| --- | --- | --- | --- |
| Devices | KITL(Realtek) | 10 |  |
|  | Ser1 | 11 |  |
|  | Ser2 | 11 |  |
|  | SerIsa | 10 | SerialIsa ports doesn’t work, fail at setting baud rate |

2) I have checked for MCS9901 in all three slots here are my observation:

|  | Slot Number 1 | IRQ | Remarks |
| --- | --- | --- | --- |
| Devices | KITL(Realtek) | 10 | Here both the Isa and the standard com ports work fine |
|  | Ser1 | 11 |  |
|  | Ser2 | 11 |  |
|  | SerIsa | 11 |  |

|  | Slot Number 2 | IRQ | Remarks |
| --- | --- | --- | --- |
| Devices | KITL(Realtek) | 10 |  |
|  | Ser1 | 11 |  |
|  | Ser2 | 10 | Here standard port “Ser2” doesn’t work, fail at setting baud rate. |
|  | SerIsa | 11 |  |

|  | Slot Number 3 | IRQ | Remarks |
| --- | --- | --- | --- |
| Devices | KITL(Realtek) | 10 |  |
|  | Ser1 | 11 |  |
|  | Ser2 | 11 |  |
|  | SerIsa | 10 | Here SerialIsa ports don’t work, fail at setting baud rate |